Triple Linear Regression Model V1.0

Matlab Toolbox Tutorial

Huayang Cai1

1Institute of Estuarine and Coastal Research, School of Ocean Engineering and Technology, Sun Yat-sen University, Guangzhou, 510275, China

Correspondence: Huayang Cai (caihy7@mail.sysu.edu.cn)

2022/09/28

A Triple linear regression model Matlab toolbox for quantifying the impacts of the Three Gorges Dam on the spatial-temporal water level dynamics in the Yangtze River estuary

Version 1.0 - September 2022

Provided by Huayang Cai

Institute of Estuarine and Coastal Research, School of Ocean Engineering and Technology, Sun Yat-sen University

Email contacts: caihy7@mail.sysu.edu.cn

How to cite:

- Huayang Cai, Hao Yang, Pascal Matte, Haidong Pan, Zhan Hu, Tongtiegang Zhao, and Guangliang Liu (2022), Quantifying the impacts of the Three Gorges Dam on the spatial-temporal water level dynamics in the upper Yangtze River estuary, OS, submitted

How to use Triple Linear Regression Model

- Download and install Triple Linear Regression Model Matlab toolbox

Users can download the latest Triple Linear Regression Model Matlab toolbox from Github:

https://github.com/Huayangcai/Triple-Linear-Regression-Model-V1.0-Matlab-Toolbox

- Triple Linear Regression Model Demo

  - Quantify the influence of boundary conditions on alteration in water levels

First of all, you need to load the data provided by triple linear regression model Matlab Toolbox (such as Data_Yangtze_river.mat). The demo can be executed using the main program labelled by ‘Example_Yangtze.m’.

The data file ‘Data_Yangtze_river.mat’ contains 3 variables, including ‘stname’, ‘Pre_TGD’ and ‘Post_TGD’.

‘stname’ denotes the name of tidal gauging stations, including 5 columns (e.g., WH, MAS, NJ, ZJ, JY, respectively).

For instance:

'WH'

'MAS'

'NJ'

'ZJ'

'JY'

‘Pre_TGD’ and ‘Post_TGD’denotes daily data observed in Pre_TGD and Post_TGD period, respectively. The data between the 1st and the 3rd column denote the time series. The data between the 4th and the 8th column denote the water level series observed in the tidal stations mentioned above. For instance, there are 5 columns of water levels in this variable, the data in the 4th column represents the water levels in WH and the data in the 8th column represent the water levels in JY. The data in the 9th column denotes daily water level data in TSG, representing the seaward boundary conditions. The data between the 10th and the 11th column denotes daily river discharge and water level data in DT, respectively, representing the upstream boundary conditions

For instance:

1978	1	1	1.9975	1.6770	1.3629	0.9694	0.3711	0.1739	11600	3.1080

1978	1	2	1.9975	1.6770	1.3629	0.9694	0.3711	0.1739	11600	3.1080

1978	1	3	1.8342	1.4983	1.1884	0.8496	0.3259	0.1557	11900	3.0680

1978	1	4	1.8342	1.4789	1.1509	0.8376	0.2772	0.0816	12000	3.0780

1978	1	5	1.7475	1.3723	1.0551	0.7001	0.2131	-0.0016	12100	3.0580

……

And

2003	1	1	3.7550	3.2714	2.7003	2.0885	1.1158	0.9174	21900	5.7430

2003	1	2	3.7611	3.2836	2.7211	2.1383	1.1508	0.9222	21800	5.6830

2003	1	3	3.6613	3.1440	2.5489	1.9085	0.8639	0.6361	21500	5.5730

2003	1	4	3.5303	3.0561	2.5030	1.9421	0.9835	0.7860	21200	5.4030

2003	1	5	3.4166	2.9452	2.3915	1.8161	0.8440	0.6544	20800	5.2630

……

The syntax of the main subroutine is illustrated below:

[alpha,beta,gamma,Z_0]=MLR(Z,[Q/std(Q),Z_down/std(Z_down),Z_up/std(Z_up)]);

Descriptions of the inputs:

Z: daily water level data used for triple linear regression analysis

Q: daily river discharge data used for triple linear regression analysis

Z_down: daily water level data at seaward station used for triple linear regression analysis

Z_up: daily water level data at upstream station used for triple linear regression analysis

Descriptions of the outputs:

alpha: linear regression coefficient for river discharge term

beta: linear regression coefficient for water level at seaward station

gamma: linear regression coefficient for water level at upstream station

Z_0: linear regression coefficient for constant term

The code is demonstrated below, you can copy and run it. The instructions introduced above are also listed in the main program.

clc,clear

clear all

close all

load Data_Yangtze_river.mat   % Load the example.

%******************inputs***************** Pre-TGD (1978-1984)

Q=Pre_TGD(:,10); % River discharges observed at the Datong (DT) hydrological station

Z_up=Pre_TGD(:,11);% Water levels observed at the upstream boundary of the estuary (DT)

Z_down=Pre_TGD(:,9);% Water levels observed at the seaward boundary of the estuary (TSG)

Z_01=[];alpha1=[];beta1=[];gamma1=[];

%% calibration_Pre_TGD

for i=1:size(stname,1)

Z=Pre_TGD(:,i+3);% Water levels observed at the gauging station

[alpha,beta,gamma,Z_0]=MLR(Z,[Q/std(Q),Z_down/std(Z_down),Z_up/std(Z_up)]); %Linear regression coefficients that are determined from the observed data according to a least-squares fit technique.

z(i,:)=Z; % inverse matrix of water levels observed in Pre_TGD

for j=1:length(Q)

wl(i,j)=Z_0+alpha*Q(j)/std(Q)+beta*Z_down(j)/std(Z_down)+gamma*Z_up(j)/std(Z_up); % A simple triple linear regression model

end

RMSE_pre_station(i)=sqrt(nanmean((Z-wl(i,:)').^2));% Root mean square error

Z_01=[Z_01,Z_0];alpha1=[alpha1,alpha];beta1=[beta1,beta];gamma1=[gamma1,gamma]; % save the parameters

end

std_z_pre=std(z');

figure1=figure;

subplot(321)

Calibration(wl(5,:),z(5,:),1)

ylabel('Pre. \itZ\rm  (m)')

text(-0.1,2.5,'(a) JY')

subplot(322)

Calibration(wl(4,:),z(4,:),1)

text(0.5,5,'(b) ZJ')

subplot(323)

Calibration(wl(3,:),z(3,:),1)

ylabel('Pre. \itZ\rm  (m)')

text(0.8,7,'(c) NJ')

subplot(324)

Calibration(wl(2,:),z(2,:),1)

xlabel('Obs. \itZ\rm  (m)')

text(1,8,'(d) MAS')

subplot(325)

Calibration(wl(1,:),z(1,:),1)

ylabel('Pre. \itZ\rm  (m)')

xlabel('Obs. \itZ\rm  (m)')

text(1.2,9.6,'(e) WH')

%%

Q=[];Z_down=[];Z_up=[];Z=[];z=[];wl=[];

%******************inputs*****************  Post-TGD (2003-2014)

Q=Post_TGD(:,10); % River discharges observed at the Datong (DT) hydrological station

Z_up=Post_TGD(:,11);% Water levels observed at the upstream boundary of the estuary

Z_down=Post_TGD(:,9);% Water levels observed at the seaward boundary of the estuary

Z_02=[];alpha2=[];beta2=[];gamma2=[];

%% calibration_Post_TGD

for i=1:size(stname,1)

Z=Post_TGD(:,i+3);% Water levels observed at the gauging station

[alpha,beta,gamma,Z_0]=MLR(Z,[Q/std(Q),Z_down/std(Z_down),Z_up/std(Z_up)]); %Linear regression coefficients that are determined from the observed data according to a least-squares fit technique.

z(i,:)=Z; % inverse matrix of water levels observed in Post_TGD period

for j=1:length(Q)

wl(i,j)=Z_0+alpha*Q(j)/std(Q)+beta*Z_down(j)/std(Z_down)+gamma*Z_up(j)/std(Z_up); % A simple triple linear regression model

end

RMSE_post_station(i)=sqrt(nanmean((Z-wl(i,:)').^2));% Root mean square error

Z_02=[Z_02,Z_0];alpha2=[alpha2,alpha];beta2=[beta2,beta];gamma2=[gamma2,gamma]; % save the parameters

end

std_z_post=std(z');

subplot(321)

Calibration(wl(5,:),z(5,:),2)

subplot(322)

Calibration(wl(4,:),z(4,:),2)

subplot(323)

Calibration(wl(3,:),z(3,:),2)

subplot(324)

Calibration(wl(2,:),z(2,:),2)

subplot(325)

Calibration(wl(1,:),z(1,:),2)

legend1=legend('Obs. pre-TGD','Obs. post-TGD','Best fit line','location','N');

set(legend1,'Position',[0.586987640997272 0.16798049734121 0.198235840297122 0.0734683281412253],'LineWidth',1,'FontSize',10);

The model performances in terms of RMSE as well as the linear regression coefficients are shown in Table 1, for both the Pre_TGD and Post_TGD periods. The RMSE values for both the Pre_TGD and Post_TGD periods were always less than 0.146 m, which suggests that the model can successfully reproduce the water level dynamics along the estuary.

Table 1  The linear regression coefficients and model performances for the 5 gauging stations along the upper Yangtze River estuary

| Stations |  | Z0 | α | β | γ | RMSE/m | Standard deviation/m |
| --- | --- | --- | --- | --- | --- | --- | --- |
| JY | Pre-TGD | 0.006 | 0.033 | 0.482 | 0.130 | 0.063 | 0.637 |
|  | Post-TGD | -0.110 | -0.075 | 0.405 | 0.260 | 0.078 | 0.581 |
| ZJ | Pre-TGD | -0.024 | 0.407 | 0.433 | 0.412 | 0.120 | 1.228 |
|  | Post-TGD | -0.136 | 0.109 | 0.359 | 0.662 | 0.121 | 1.106 |
| NJ | Pre-TGD | -0.190 | 0.508 | 0.314 | 0.919 | 0.131 | 1.718 |
|  | Post-TGD | -0.422 | 0.133 | 0.287 | 1.109 | 0.144 | 1.505 |
| MAS | Pre-TGD | -0.188 | 0.519 | 0.261 | 1.265 | 0.138 | 2.025 |
|  | Post-TGD | -0.401 | 0.217 | 0.240 | 1.324 | 0.146 | 1.760 |
| WH | Pre-TGD | -0.243 | 0.394 | 0.175 | 1.801 | 0.114 | 2.355 |
|  | Post-TGD | -0.445 | 0.170 | 0.170 | 1.707 | 0.108 | 2.030 |

Figure 1  Comparison between predicted and observed daily averaged water levels for both the pre-TGD and post-TGD periods at different gauging stations along the YRE: (a) Jiangyin (JY), (b) Zhenjiang (ZJ), (c) Nanjing(NJ), (d) Maanshan (MAS), (e) Wuhu (WH)